Using MUSIC-link for City of Gold Coast

In order to improve the development assessment process with regards to Water Sensitive Urban Design, Council is working with eWater to simplify the process of developing Stormwater Treatment Strategies. The use of MUSIC-link can simplify the development and assessment of MUSIC models.

As such, Council encourages proponents to utilise the MUSIC-link function when preparing MUSIC models for proposed developments within the City of Gold Coast.

When City of Gold Coast is assessing a development proposal, we must check that it complies with our Water Sensitive Urban Design policies. While eWater's MUSIC software makes it possible to achieve compliance with the targets and parameters, the assessment and refinement process has traditionally been iterative and time-consuming.

The main benefit of MUSIC-link is that developers and consultants can design their stormwater management infrastructure and then immediately validate it (within the software) to ensure that parameters fall within the limits of City of Gold Coast's requirements. On receiving a submitted design and related validation report, our Development Assessment team can immediately check for compliance with our standards. If the design is compliance, it can be passed onto engineers for detailed evaluation.

MUSIC-link streamlines the process of achieving a match between the City of Gold Coast's specific guidelines and urban developers' water sensitive designs. This shortcut bypasses the traditionally iterative process, where assessing authorities returned models to developers to be amended and re-submitted until they are suitable for engineering evaluation.

MUSIC-link allows City of Gold Coast to:

- Apply a simple, robust and quicker process of WSUD assessment, helped by the compliance report that MUSIC-link provides with designs;

- Communicate clearly and directly with developers and designers, providing locally specific WSUD requirements and modelling parameters;

- Provide increased levels of redundancy for model assessment; and

- Build capacity and support policy redundancy to support longevity.

Using MUSIC-link

MUSIC-link forms part of the standard MUSIC v6 interface, and can be accessed using the MUSIC-link tab (shown below) in the main interface.

MUSIC-link tab

Validate MUSIC-link Model

Working with MUSIC-link is a two-step process:

- Create and run your MUSIC model; and

- Validate you model against the set of pre-defined parameters for a particular assessing authority.

Creating a MUSIC model

When creating a MUSIC model for inclusion in the validation process, you can choose one of two methods: either create a standard model, or directly create a MUSIC-link model, as described below.

Create a standard model

- Choose a standard meteorological template that is available with MUSIC;

- Build and run your MUSIC model;

- In the MUSIC-link tab, click Configure and choose Yes to convert the existing model to a MUSIC-link model;

- Choose City of Gold Coast and the associated meteorological template from the MUSIC-link Configuration window that opens (as per below diagrams); and

- Then, initiate the validation process. Refer to Validating the MUSIC-link model for details.

Directly create a MUSIC-link model

- Choose New MUSIC-link from the main MUSIC tab;

- Choose the City of Gold Coast option from the drop down list and the associated meteorological template (as shown below);

- Build your model; and

- Then, initiate the validation process. Refer to Validating the MUSIC-link model for details.

Note: When using MUSIC-link, nodes with import flows do not contribute to the total areas or the impervious percentage calculations

Access to MUSIC-link with the names of assessing authorities from main MUSIC tab (only a partial list is shown in this figure)

Self-Validating the MUSIC-link model (Prior to lodgement with City of Gold Coast)

Ensure that your MUSIC model has been created with City of Gold Coast inputs prior to validating it. Once you have created your model:

- Choose the MUSIC-link tab and click Run MUSIC-link; and

- Once the model has been run, the MUSIC-link window appears showing the validation report (see below).

Validation report

A report is generated on completion of a validation run (as shown below). The top half of the report provides details of the validation result. It shows the compliance status of each parameter, which indicates whether the result in within the allowable range. An unchanged result indicates that this value has not been modified by the user (default value used during model run), and hence, it has no effect on overall compliance.

You can now do the following with the report:

- Click Create Report to expand a form, where you can enter details of the project & associated notes as well as contact details for those who are submitting the model & report. A report will only be generated (and can be saved in a PDF format) if all the parameters are compliant, or you have provided an explanation to justify why parameters are non-compliant (using the Comment field)

- Click Next Steps to expand the report for City of Gold Coast lodgement Instructions.

Compliance Status

Range of allowable values

Passed

Failed

Unchanged

Lodgement of your MUSIC model and MUSIC-link Report to City of Gold Coast

If the MUSIC model meets the reportable parameter requirements for City of Gold Coast’s setup configuration and targets, proceed as follows:

- Create/Generate a City of Gold Coast MUSIC-link PDF report for submission with your model;

- Submit your self-validated MUSIC model along with the City of Gold Coast MUSIC-link PDF report to City of Gold Coast as part of your Development Application.

If the parameters and/or targets are outside the required ranges and you believe the use of these parameters or the output results can be justified, please follow the steps below:

- Generate a City of Gold Coast MUSIC-link PDF report for submission with your model. NOTE: Any parameter breaches must be justified for the model to be considered by City of Gold Coast;

- Provide reasons for model assumptions that are outside of default parameter ranges in the Comments section; and/or

- Submit your self-validated MUSIC model along with the City of Gold Coast MUSIC-link PDF report to City of Gold Coast as part of your Development Application.

All material submitted must be in accordance with City of Gold Coast’s DA submission requirements.

Further Assistance:

If you require further assistance with utilising MUSIC-link City of Gold Coast, please contact eWater support on 1300 92 837 support@ewater.com.au or City of Gold Coast on 07 5582 8866, during office hours.